# VLSI DESIGN INTERNSHIP – TASK 2 REPORT

## Introduction to Verilog HDL and RTL Design of Basic Combinational Circuits

# COVER PAGE

VLSI DESIGN INTERNSHIP

### TASK – 2

## Introduction to Verilog HDL and RTL Design of Basic Combinational Circuits

### Submitted By

Name : Ruthran G

Register Number : 513123106080

College : Thanthai Periyar Government Institute of Technology

# 1. OBJECTIVE

The objective of this task is to understand Verilog HDL, implement combinational logic circuits using RTL modeling, write testbenches, simulate the design, and verify outputs using waveform analysis.

# 2. INTRODUCTION

Verilog HDL is a Hardware Description Language used to model, design, and verify digital electronic systems. It is one of the most widely used languages in VLSI chip design.

RTL (Register Transfer Level) describes the behavior of digital circuits in terms of data flow between registers and the logic operations performed on that data.

# 3. VERILOG HDL

## Features

- Hardware Description Language

- Supports combinational and sequential circuits

- Used in FPGA and ASIC design

- Easy simulation and verification

### Basic Module Structure

module gate_name(
input A,
input B,
output Y
);

assign Y = A & B;

endmodule

# 4. RTL DESIGN

RTL (Register Transfer Level) is a design abstraction used in digital circuit design where data transfer between registers and logical operations are described.

### Advantages

- Faster hardware development

- Easy simulation

- Technology independent

- Synthesizable design

# 5. IMPLEMENTATION OF LOGIC GATES

## AND Gate

### Verilog Code

module and_gate(input A,input B,output Y);
assign Y=A&B;
endmodule

Screenshot

## OR Gate

module or_gate(input A,input B,output Y);
assign Y=A|B;
endmodule

Screenshot

## NOT Gate

module not_gate(input A,output Y);
assign Y=~A;
endmodule

Screenshot

## NAND Gate

module nand_gate(input A,input B,output Y);
assign Y=~(A&B);
endmodule

Screenshot

## NOR Gate

module nor_gate(input A,input B,output Y);
assign Y=~(A|B);
endmodule

Screenshot

## XOR Gate

module xor_gate(input A,input B,output Y);
assign Y=A^B;
endmodule

Screenshot

# 6. HALF ADDER

### Logic

Sum = A ^ B

Carry = A & B

### Verilog Code

module half_adder(
input A,
input B,
output Sum,
output Carry
);

assign Sum=A^B;
assign Carry=A&B;

endmodule

### Truth Table

| A | B | Sum | Carry |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

Screenshot

# 7. FULL ADDER

### Logic

Sum = A ^ B ^ Cin

Carry = (A&B) | (B&Cin) | (A&Cin)

### Verilog Code

module full_adder(
input A,
input B,
input Cin,
output Sum,
output Carry
);

assign Sum=A^B^Cin;
assign Carry=(A&B)|(B&Cin)|(A&Cin);

endmodule

### Truth Table

| A | B | Cin | Sum | Carry |
| --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 0 |
| 0 | 0 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 0 |
| 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 1 | 0 | 1 |
| 1 | 1 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 1 |

Screenshot

# 8. TESTBENCH

A testbench is used to verify the functionality of a Verilog module by applying different input combinations and observing the outputs.

### Example

initial begin

A=0;B=0;
#10;

A=0;B=1;
#10;

A=1;B=0;
#10;

A=1;B=1;
#10;

end

Screenshot

# 10. RESULTS

Successfully implemented:

- AND Gate

- OR Gate

- NOT Gate

- NAND Gate

- NOR Gate

- XOR Gate

- Half Adder

- Full Adder

All outputs matched their expected truth tables during simulation.

# 11. CONCLUSION

Task-2 provided practical knowledge of Verilog HDL programming and RTL modeling. Basic combinational circuits, Half Adder, and Full Adder were implemented successfully. Testbenches were written to verify each design, and waveform analysis confirmed the correctness of the circuits.

# 12. REFERENCES

- Verilog HDL Reference Manual

- RTL Design Notes

- Icarus Verilog Documentation

- GTKWave Documentation

- MainCrafts Internship Task-2 PDF